SDS-PAGE and transfer

Sample buffer:		50 mM Tris-HCl (pH 6.8)

10% glycerol

2% SDS

100 mM DTT

distilled water to 1 ml

5x Running buffer:	15.1 g Tris base

94 g glycine (electrophoresis grade)

25 ml 20% SDS

distilled water to 1000 ml (pH should be 8.3)

1. Load 10 or 15 g of total protein in sample buffer (incubated for 3 min. at 95°C) on 8%-15% SDS-PAGE minigel with stacking gel on top.

2. Run under 15 mA per 1 gel, until bromphenol blue reaches the separating gel. Increase the current to 30 mA per gel and run until bromphenol blue reaches the edge of the gel.

3. Remove gels, cut off the stacking gel and mark the orientation by cutting one corner.

4. Transfer proteins from the gel to PVDF membrane using the Transblot Turbo transfer system (BioRad), according to the manual

Immunoblotting

1x TBST buffer:	50 mM Tris-HCl (pH 7.8)

150 mM NaCl

1 mM EDTA (pH 8)

0.1% Tween-20

(10x TBST can be made)

Blocking buffer:	1x TBST

5% BSA (fraction V, Sigma)

5% nonfat dry milk

(keep refrigerated, use within 24 hours !!)

1. Put the membranes into 50 ml Falcon tubes with 2 ml of blocking buffer.

2. Block overnight rotating at 4C.

3. Add a primary antibody in appropriate dilution.

4. Incubate for 2 h at RT rotating.

5. Discard the primary and wash membranes 5 times for 10 min per each change with shaking in fair amount (50 ml) of TBST at RT.

6. Put membranes into 50 ml Falcons with 2 ml of blocking buffer containing peroxidase conjugated secondary antibody (Jackson Immunores.) (dilution 1:5000).

7. Incubate for 1 h at RT rotating.

8. Wash 5 times in TBST as in step 5.

Detection with chemiluminescense (home-made ECL protocol)

Frozen aliquots:

Luminol (Sigma A8511): 250 mM in DMSO

p-coumaric acid (Sigma C9008): 90 mM in DMSO

Per 10 ml of reagens,

1. Mix solution 1:

4.5 ml		water

500 l		1 M Tris-HCl (pH 8.5)

50 l		250 mM luminol

22 l		  90 mM p-coumaric acid

and solution 2:

4.5 ml		water

500 l		1 M Tris-HCl (pH 8.5)

3 l		30% H2O2

2. Combine solutions 1 and 2 just before use.

3. Incubate membrane for 2-5 min and immediately wrap and expose (the signal rapidly decreases and is completely dead after 30-45 minutes !!!).